SSD Documentation

SSD Cables and Connectors

Version Number : 3

04-july-02

1 Cables and connectors running from the south platform to the TPC wheel	3

STAR- SSD  Connectors specifications	3

STAR- SSD cables specifications	3

2 cables and connectors running on the SVT/SSD support cones :	4

2.1 Cables	4

2.2 Connector for power, sense and high voltage cable :	4

2.3 connector for the signal cable :	5

# 1 Cables and connectors running from the south platform to the TPC wheel

STAR- SSD  Connectors specifications

| Connectors | Cable / Location | Manufacturer Part Number | Material | Max Current | Operating Voltage (Max) | Voltage Rating | Temperature Rating | Flammability Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inside Rack Cable Low Voltage | RKLVxx PS, Distr Crate | Amphenol 777-RR-B25P | glass-filled thermoplastic | 2A | 5V | 500V | -55°C 105°C | ULV94V-0 |
| Inside Rack Panel Low Voltage | RKLVxx PS, Distr Crate | Amphenol 177-RR-B25S | glass-filled thermoplastic | 2A | 5V | 500V | -55°C 105°C | ULV94V-0 |
| Inside Rack Cable High Voltage | RKHVxx PS, Distr Crate | Amphenol 777-RR-C37P | glass-filled thermoplastic | 1mA | 50V | 500V | -55°C 105°C | ULV94V-0 |
| Inside Rack Panel High Voltage | RKHVxx PS, Distr Crate | Amphenol 177-RR-C37S | glass-filled thermoplastic | 1mA | 50V | 500V | -55°C 105°C | ULV94V-0 |
| Inside Rack Panel Mixed Voltage | FMVxx,SMVxx Rack | AMP CPC 206838-1 | glass-filled thermoplastic | 2A | 50V | 1500V | -55°C 105°C | ULV94V-0 |
| Cable Mixed Voltage | FMVxx,SMVxx TPC Wheel | AMP CPC 206837-1 | glass-filled thermoplastic | 2A | 50V | 1500V | -55°C 105°C | ULV94V-0 |

STAR- SSD cables specifications

| Cables | Nb cond | AWG | Manufacturer Part Number | Material | Max Current | Operating Voltage (Max) | Voltage Rating | Temperature Rating | Flammability Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inside Rack Low & High Voltage (RKLVxx, RKHVxx) | 27 | 22 | HELUKABEL LiY-CY 20070 | Copper PVC | LV : 1A HV : 1mA | LV: 5V HV : 50V | 300V | -30°C 105°C | IEC 332-1 |
| Mixed Voltage Force (FMVxx) | 12 | 17 | HELUKABEL LiY-CY 16483 | Copper PVC | LV : 1A HV : 1mA | LV: 5V HV : 50V | 600V | -30°C 90°C | IEC 332-1 |
| Mixed Voltage Sense (SMVxx) | 8 | 22 | HELUKABEL LiY-CY 20061 | Copper PVC | 1mA | 50V | 300V | -30°C 105°C | IEC 332-1 |
| Control (CTRxx) | 27 | 22 | HELUKABEL LiY-CY 20070 | Copper PVC | - | 5V | 300V | -30°C 80°C | IEC 332-1 |

# 2 cables and connectors running on the SVT/SSD support cones :

## 2.1 Cables

| Cable name | AWG | Max current | Operating Voltage | Manufacturer & part numb. | material | Voltage rating | Operating Temperature | Flammability rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power cables low volt. side | 20 | 2 A | 0 V | Alcatel Lyflex | PVC Copper | 500 V | -10 C to + 60 C | NF C 32-070 C2 CEI 332-1 |
| Power cables high volt. side | 20 | 2 A | 50 V | Alcatel Lyflex | PVC Copper | 500 V | -10 C to + 60 C | NF C 32-070 C2 CEI 332-1 |
| Sense cables low volt. side | 24 | 0 A | 0 V | Alpha wire 5599/5 | PVC Copper Aluminium | 300 V | -20 C to +80 C | UL VW1 |
| Sense cables high volt. side | 24 | 0 A | 50 V | Alpha wire 5599/5 | PVC Copper Aluminium | 300 V | -20 C to +80 C | UL VW1 |
| Signal & power cable | 28 | 300 mA | 0-5V | 3M KU-KM PVV-SB | PVC Copper | 300 V | -20 C to +60 C | UL VW1 |
| High voltage cable | 24 | 1mA | 0-50V | Alpha wire 5092 | PVC Copper | 300 V | -20 C to +80 C | UL VW1 |

## 2.2 Connector for power, sense and high voltage cable :

At the large end of the cone, only one connector is used per group of cable. That means the power cable, the sense cable and the ‘high voltage cable (50 V) are connected together.

- AMP : M series 14 position

housings material : 				phenolic

temperature rating :				-55C to +150C

flammability rating : 				UL94V-0

dielectric withstanding voltage (sea level) :	AC 900 V,rms

current rating :				13 A

At the small end of the cone, two types of connector are used the group of cable.

- Taitek : power cable and HV cable

type :		2.54mm header housing and crimp terminal

part number					254WS08G

number of pins				8

housing material : 				Nylon

temperature rating :				-40 C to +105C

flammability rating :				UL 94V-2

dielectric withstand voltage :			AC 1200 V rms

operating voltage				AC 250 V rms

current rating					5 A

- FCI : sense cable

type :						HE13

part number :					6110 03 2

number of pins : 				6, double row

temperature rating				-55C to +125 C

flammability rating				UL 94V-0

dielectric withstanding voltage		AC 1000 V rms

current rating					3 A

## 2.3 connector for the signal cable :

- Harting : SCSI connector

type 						SCSI connector

part number					60 03 068 5100

housing material				thermoplastic doped with glass

shield material					brass

number of pins				68 pins

temperature rating				-55C to +105 C

flammability rating				UL 94V-0

dielectric withstanding voltage		AC 750 V rms

operating voltage				AC 240 V rms

operating current				1A

Annexe