SSD cabling overview and documentation

The SSD cabling is foreseen as the following :

HV+

HV-

HV crate

Power supply

-2V

0V

+2V

Power supply

-2V

0V

+2V

hybrid

ADC

hybrid

ADC

Optocoupleur

Optocoupleur

Readout

Power supply

0 V

+5V

Power supply

0V

+5V

STAR ground

20

20

20

20

20

0V

0V

2

2

Power supply

0V

+5V

Power supply

0V

+5V

Equivalent to one ladder

Readout

Power cables conductors

High voltage cables conductors

Sense wires

flat cable

Included in the same SCSI cable

STAR ground

So for one cone the summary of cables is the following :

Power supply	 and High voltage cables

High voltage side	7 conductors		(+2V,0V),(0V,-2V),(0V,+5V), no used	10

Low voltage side	7 conductors		(+2V,0V),(0V,-2V),(0V,+5V),no used-	10

Low voltage side	2 conductors		HV+,HV-					10

Sense wires

High voltage side 	5 conductors		+2V,0V,0V,-2V,+3V				10

Low voltage side 	5 conductors		+2V,0V,0V,-2V,+3V				10

Those conductors can be grouped as following :

20 power and high voltage cables with 7 conductors (AWG 20)

20 sense cables with 5 conductors (AWG 24)

10 High voltage cable 2 conductors (AWG24)

Signal cable

Signal			68 conductors								2

This cables includes non floating power supply (+5V,0V), control signal (JTAG+Address bus+Latchup info + Test + Hold) and data.

Connector Pin-out :

At the large end of the cone, one connector is used for one group of cable corresponding to one ladder. The pin-out of this connector is the following :

AMP M series 14 positions :

A : black	HT+

B : brown	HT-

C : black	Sense +2V

D : brown	Sense 0V

E : red	Sense -2V

F : orange	Sense 0V

H : yellow	NC

J : bleu		+2V

K : brown	0V

L : green	-2V

M : gray	0V

N :yellow	NC

P : pink	+5V

R : white	0V

rear view of the connector

Few comments about this pin-out :

HT cable :

- gauge 24

- connected to pins A and B

- no HT cable in group W1 to W10 and E11 to E20

Sense cable :

- gauge 24

- connected from pin C to pin F

Power cable :

- gauge 18

- connected from pin J to pin R

Pin :

- pin H and pin N are connected to a conductor but are not used.

- pin P and pin R are female

At the small end of the cone, the power cable and the high voltage cable are connected to the same device, a Taitek connector, and the sense cable is connected to a FCI connector.

- Taitek : power and HV connector

1 : grey	0V

2 : green	-2V

3 : brown	0V

4 : blue	+2V

5 : white	0V

6 : pink	+5V

7 : brown	HT-

8 : black	HT+

- FCI : sense connector

1 : black	+2V

2 : brown	0V

3 : red	-2V

4 : orange	0V

5 : yellow	not attributed

6 : not connected