SVT+SSD alignment

1. Definitions:

Track parameters:  t = (xv, λ,φ,ξ≡1/pT) = (xv,d)

- xv = (xv,yv,zv) - primary vertex ;

- λ - angle between track and xy-plane (dip) and

- φ - angle between track projection on xy-plane and x-axis;

- ξ = 1/pT - signed inverse momentum,

- curvature 1/ρ[cm] = - 2.9979251×10-4 Bz[kG]/pT[GeV/c].

Helix:

- x(s) = xv, + ρ × (cos(φ + s×cos(λ)/ρ) – cos(φ));

- y(s) = yv, + ρ × (sin(φ  + s×cos(λ)/ρ) – sin(φ));

- z(s) = zv, + s×sin(λ);

s – distance in space along the track to the given point x from xv.

Measurement:

m = (u,v,w≡0) – measurement in local coordinate system;

Prediction:

p = f(t,a) = f(t0,a0) + H × (t - t0) + A × (a - a0) =

f(t0,a0) + V × (x - xv) + D × (d – d0) +  A × (a - a0)

where Jacobians:

H = ∂f/∂t(t0,a0); A = ∂f/∂a(t0,a0);

V = ∂f/∂xV(t0,a0); D = ∂f/∂d(t0,a0);

Measurement plane (ideal) a0 =(R, t) is defined by translation vector (t) and rotation matrix (R).

l = R × (r – t),  l and r are local and global coordinates, respectively.

R=r0r3r6r1r4r7r2r5r8≡d0t0n0d1t1n1d2t2n2

where d,t and n are drift, transverse and normal direction in SVT and SSD wafer position definitions.

Prediction on ideal plane:

p = (up,vp,wp≡0) = l = R × (r – t)

Misalignment is defined by Δt and ΔR:

Δt = (Δu,Δv,Δw);

and

ΔR=1ΔγΔβ−Δγ1Δ−Δβ−Δα1α

Corrected prediction:

q = (uq,vq,wq≡0) =ΔR ×(R × (r – t) - Δt) = ΔR×(px  - Δt),

prediction with accounting of misalignment i.e. a = (ΔR×R, t+ R-1×Δt).

Condition wq  = 0 in straight line approximation

px = p + h×j;

ī∙q(h) = ī ∙(ΔR×px  -  ΔR ×Δt) =  ī ∙ ΔR × (p - Δt) +  h×ī ∙(ΔR×j) =  0;

where: h – step,

j = (tu,tv,1)/√(1 + tu2 + tv2 ) = (cos(θ), cos(ψ),cos(υ)) - direction cos at ideal measurement plane

and ī = (0,0,1).

h = - ī ∙ ΔR × (p - Δt)  ī ∙(ΔR×j) =

= -0011ΔγΔβ−Δγ1Δ−Δβ−Δα1αu−Δuv−Δv−Δw0011ΔγΔβ−Δγ1Δ−Δβ−Δα1αcos(θ)cos(ψ)cos(ϑ)=

= - (u−Δu)+Δγ(v−Δv)−ΔβΔw−Δγ(u−Δu)+(v−Δv)−ΔαΔw−Δβ(u−Δu)−Δα(v−Δv)−Δw001cos(θ)+Δγcos(ψ)+Δβcos(ϑ)−Δγcos(θ)+cos(ψ)+Δαcos(ϑ)−Δβcos(θ)−Δαcos(ψ)+cos(ϑ) =

= (Δβ(up-Δu) + Δα(vp-Δv) + Δw)/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ)) ≈ Δw/cos(υ)

uvq=1ΔγΔβ−Δγ1Δα×u−Δu+hcos(θ)v−Δv+hcos(ψ)−Δw+hcos(ϑ)p==(u−Δu+hcos(θ))+Δγ(v−Δv+hcos(ψ))+Δβ(−Δw+hcos(ϑ))−Δγ(u−Δu+hcos(θ))+(v−Δv+hcos(ψ))+Δα(−Δw+hcos(ϑ))

≈ u−Δu+h⋅cos(θ)+Δγ(v−h⋅cos(ψ))+Δβ⋅h⋅cos(ϑ)−Δγ(u+h⋅cos(θ))+v−Δv+h⋅cos(ψ)+Δα⋅h⋅cos(ϑ)

where ‘q’ and ‘p’ correspond to real and ideal predictions.

Derivatives corrected prediction with respect to uncorrected ones:

∂q/∂p = I, i.e. ignore second order effects

Derivatives corrected prediction with respect to alignment parameters:

l = (Δu,Δv,Δw, Δα, Δβ, Δγ);

h = (Δβ(up-Δu) + Δα(vp-Δv) + Δw)/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ)) ≈ Δw/cos(υ)

∂h/∂(Δu) = -Δβ/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ)) ≈ 0;

∂h/∂(Δv) = -Δα/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ)) ≈ 0;

∂h/∂(Δw)=    1/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ)) ≈ 1/cos(υ);

∂h/∂(Δα) = ((vp-Δv)(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ)) - (Δβ(up-Δu) + Δα(vp-Δv) + Δw)(-cos(ψ)))/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ))2≈

vp /cos(υ);

∂h/∂(Δβ) = ((up-Δu)(cos(υ)- Δβ×cos(θ) - Δα× cos(ψ)) - (Δβ(up-Δu) + Δα(vp-Δv) + Δw)(-cos(θ)))/(cos(υ)- Δβ×cos(θ)  - Δα× cos(ψ))2≈

up /cos(υ);

∂h/∂(Δγ) = 0;

∂h/∂l = (0,0,1,vp /cos(υ), up /cos(υ),0);

q    ≈ u−Δu+h⋅cos(θ)+Δγ(v−h⋅cos(ψ))+Δβ⋅h⋅cos(ϑ)−Δγ(u+h⋅cos(θ))+v−Δv+h⋅cos(ψ)+Δα⋅h⋅cos(ϑ)

∂(uq)/∂(Δu) = -1 + ∂h/∂(Δu) ×cos(θ) = -1;                             ∂(vq)/∂(Δu) = 0;

∂(uq)/∂(Δv) =  0;                                                                    ∂(vq)/∂(Δv) = -1;

∂(uq)/∂(Δw) = ∂h/∂(Δw) ×cos(θ)  = cos(θ)/cos(υ)  ;                    ∂(vq)/∂(Δw) = ∂h/∂(Δw) ×cos(ψ)  = cos(ψ)/cos(υ) ;

∂(uq)/∂(Δα) = ∂h/∂(Δα) ×cos(θ)    = vp ×cos(θ)/cos(υ);          ∂(vq)/∂(Δα) ≈  ∂h/∂(Δα) ×cos(ψ)   = vp cos(ψ)/cos(υ) ;

∂(uq)/∂(Δβ) ≈ h×cos(υ)+ ∂h/∂(Δβ)×cos(θ)≈up×cos(θ)/cos(υ);∂(vq)/∂(Δβ) ≈  ∂h/∂(Δβ) ×cos(ψ)   ≈ up cos(ψ)/cos(υ);

∂(uq)/∂(Δγ) ≈  vp  - h×cos(ψ) ≈ vp ;                                        ∂(vq)/∂(Δγ) ≈  -(up -h×cos(θ)) ≈ -up ;

∂(q)/∂l = ∂(uq,vq)/∂l = =

=

j = (tu,tv,1)/√(1 + tu2 + tv2 ) = (cos(θ), cos(ψ),cos(υ))

Functional:

mij– measurement for j-th hit associated with i-th track in event.

hij = fj(ti,aj) = fj(xi,di,aj) = f j (x0i, d0i,a0j) + Vij× (xi – x0iv) + D ij× (di – d0i) +  A ij× (aj - a0j)

Gij- inverse error matrix

Φ = χ2 =   (Τ×χ – ψ)+×G×(Τ×χ – ψ)  =

ij (mij– hij)+ ×Gij× (mij – hij) =

ijεij+ ×Gij× εij;

ψ  = εij = mij - f j (x0i, d0i,a0j) = εij0 - f j (x0i, d0i,a0j) + Vij× (xi – x0iv) + D ij× (di – d0i) +  A ij× (aj - a0j)

χ = (xi – x0iv, di – d0i, aj - a0j) = (Δxi,Δdi,Δaj);

εij = εij0 – (f j (x0i, d0i,a0j) + Vij× (xi – x0iv) + D ij× (di – d0i) +  A ij× (aj - a0j));

½ ∂ Φ /∂( xi, di, aj) = Τ+×G×Τ×χ  - Τ+×G× ψ = 0; χ = (Τ+×G×Τ)-1 ×Τ+×G× ψ;

½ ∂ Φ /∂( xi, di, aj) = ijεij+ ×Gij× ∂εij/∂( xi, di, aj) = 0

½ ∂ Φ /∂( xi) = ijεij+ ×Gij× ∂εij/∂( xi);

Condition to stay on measurement plane:

u’ = u +