- valid from 7/14/2009

- STAR TPC GAS SYSTEM CHECK LIST for CH4 Purge

(To be filled out every 2 hours from ________________  to ________________ )

Rack 1

| Sensor | Function | Value | Range | Comments |
| --- | --- | --- | --- | --- |
| FM2 | Fresh Methane Flow (slaved) |  | 10.2 -11.2 lpm |  |
| FM3 | Fresh Argon Flow to TPC |  | 85-105 lpm |  |
| PT8(PI6) | TPC Pressure |  | 250-450 bar |  |

- Rack 2

| Sensor | Function | Value | Range | Comments |
| --- | --- | --- | --- | --- |
| M4 | Fresh Methane Content |  | 9.5-10.5% |  |

Rack 3

| Sensor | Function | Value | Range | Comments |
| --- | --- | --- | --- | --- |
| M3a | Methane Meter |  | 9.5-10.5% |  |
| M1 | Oxygen Meter |  | 5 - 80 ppm |  |
| PI9 | Methane Delivery Pressure |  | 13.0-17.0 PSIG | Should be less than PI8 |
| PI8 | Argon Delivery Pressure |  | 15.0 – 19.0 PSIG |  |

Rack 4

| Sensor | Function | Value | Range | Comments |
| --- | --- | --- | --- | --- |

Operator__________________________		Date & Time____________

Alexei Lebedev	Jim Thomas

Office:    (631)  344-3101	Office: 	(631)  344-3918

Cell:       (631)  255-4977	Cell:    	(510)  759-4936

Home:    (631)  821-2838	Home:  	(631)  928-8661